# ASV Simulator

Quick Start — from nothing to a survey running, in about twenty minutes

Everything here runs against the SIMULATOR, so there is no vessel to damage and nothing to get wet. The console drives a real vessel identically, so what you learn here transfers directly. Section references point into the Operations Manual, which has the depth.

# 1  Run it

Python 3 and its standard library. No packages to install, no build step.

cd <project>

python asv_console.py --sim

A browser opens on the console. It also opens a second CONTROLS window — drag that to another screen if you have one, or add --single-window to keep everything together.

The vessel appears on a live nautical chart at its configured operating area, already connected to the simulator. Give it a few seconds: on the first position fix the console fetches the chart for the area and builds its keep-out model.

WATCH THIS FIRST  The vessel-status card's Nogo row tells you when the chart is ready. It counts seconds while reading, then reports the zone count and this vessel's depth floor. Do not plan until it has landed. (Ops manual 6.2.1)

# 2  Get your bearings

| Where | What it is |
| --- | --- |
| Top bar | Position, speed, heading, energy, and the SAFETY STATE — it reads SAFE until you arm |
| Right-hand card | The detailed vessel state, plus a RUN block once anything is planned |
| Button column | Chart layers and planning modes |
| Bottom bar | Run parameters and the commands |
| ? bottom-left | The built-in quick start card |

Zoom with + / − (on the command bar, left of ARM), drag to pan, BOAT re-centers on the vessel. Turn on NOGO to see the keep-out zones the vessel will refuse to cross — that overlay is the single most useful thing on screen while you are learning. (Ops manual 4)

# 3  Your first command — a Go-To

Nothing moves until you arm. That is the whole safety model in one sentence.

- Press Arm. The safety pill changes from SAFE.

- RIGHT-CLICK a point on open water a few hundred meters away and choose GO-TO HERE. The chart's right-click menu is where the point commands live — Go-To, Set Home, Spawn, and the measuring tool. Until you arm, the Go-To row is grayed and tells you why.

- Read the BANNER across the chart. It says whether the route went direct or was routed around obstacles, and how many waypoints it used.

The vessel drives there, around anything charted in the way, and station-keeps on arrival. Now try clicking a point on the far side of a pier or a shoal — the console will route a way round it, or REFUSE and pulse the offending feature with a cross. A refusal is a result, not a failure. (Ops manual 8.1, 2.4)

# 4  Your first survey

## 4.1  Draw the pattern

On the Path planning card, choose the Survey tab - with no pattern yet it arms the chart (otherwise press Draw; Esc stops) - and click three times:

- The START CORNER of the area you want covered.

- The OPPOSITE (diagonal) corner.

- A third point — its DISTANCE from the start sets the line spacing, its BEARING sets the line direction.

Parallel lines fill the box immediately. Drag any of the three points to adjust; drag the CROSSHAIR GRIP at the center to move the whole pattern without changing its shape. (Ops manual 9.1)

That is the Survey switch on PATTERN, the default. When the survey has to stay inside a set area, choose BOUNDED instead: draw the area on the chart, or press Import and pick a GeoJSON, KML or CSV file of it. The lines fill the area. (Ops manual 9.3)

## 4.2  Punch it out

Press Punch Out. The console fetches the chart for the survey area and trims every line out of land, structures, hazards and water outside your depth limits. THIS IS THE STEP THAT MAKES THE PLAN TRUSTWORTHY — read what it reports.

IF IT SAYS “UNTRACKABLE”  Your line spacing is tighter than the vessel can turn in at the plan speed. Widen the spacing or drop the speed and punch out again. (Ops manual 9.5.1)

## 4.3  Commit and run

- Add to plan — commits the lines and waypoints.

- Upload — pushes the plan to the vessel, routing the approach clear of obstacles.

- Start — the run begins.

Watch the RUN block: type, waypoint progress, distance and time to the end, percent complete. LINES opens a per-line table showing planned against actual time as the run proceeds.

# 5  Know where it will end up

The End of plan selector on the Path planning card (under Waypoints and end of plan, on every tab) decides what happens when the plan finishes — RTH (return home and station-keep), Complete (stop), Loiter (hold at the last waypoint), or Repeat (loop until you stop it).

The RUN block's End mode row tells you where THIS run will leave the vessel, from the moment it starts. With RTH selected it says so throughout — and it only says RTH when the return can actually happen. (Ops manual 11.5)

# 6  Stopping

| Control | Effect |
| --- | --- |
| Pause | Holds position, keeps the next waypoint — Start resumes |
| Stop | Aborts the run |
| E-STOP | Command-side motor cut and force-disarm. NOT the vessel's true failsafe — that is the transmitter. |
| Reset | Simulator only: power-cycle back to a clean slate |

# 7  Worth trying next

| Try | Why |
| --- | --- |
| SRC | The chart's own title block and the SURVEY CONFIDENCE under the vessel. Some water is charted far better than other water. |
| Wind / Sea / Set on the vessel card | Real wind and sea push the simulated vessel off track. Watch SET and the crab angle change as it steers out of the drift. |
| Switch vessel | The picker in the top bar. A different hull changes speeds, turn radius and the depth floor — watch the keep-out model change with it. |
| Passage tab | On the Path planning card: a long passage (diesel hulls) - drop its points, Plan passage (its two ends planned in detail first), Sail passage. A Go-To past 50 km on a diesel hull is planned as one. |
| Transit tab | On the Path planning card: draw a multi-segment transit and Follow it, when you want a specific path rather than the router's. |
| Go-To tab | Click or type a point on the Path planning card: its bearing, range and time show before Go moves anything. |
| ROC | Place a mothership, set it steaming, make it HOME — then Return-to-Home chases a moving recovery point. |
| Playback | Every session is recorded. Replay it on the same chart with a timeline of every command. |

# 8  Where to go from here

| Document | For |
| --- | --- |
| Operations Manual | The full operational picture — safety model, every control, planning depth, contingencies, checklists |
| Technical Manual | Architecture, subsystems, HTTP API, data formats, extension recipes |
| Development Guide | How the project is built and verified, and the practices that keep it honest |
| README.md | Feature-by-feature tour with the reasoning behind each one |

BEFORE A REAL VESSEL  Read the Operations Manual's safety model (section 2) and its checklists (section 16) in full. The simulator forgives everything; the water forgives nothing.